import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent DP-PAM4 Optical Communication System with JAX High-Speed ​​LMS Equalization

Welcome to the interactive simulation experiment of optical communication using pure one-dimensional real modulation (PAM4)!

This tutorial demonstrates how to use the **data-driven + hard-decision hybrid (DA/DD-LMS)** algorithm.

"""

)

return (mo,)

@app.cell

def _():

import time

import jax

import jax.numpy as jnp

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

import numpy as np

from commplax import adaptive_kernel as ak

from commplax.equalizer import CPR

from commplax import sym_map

def get_snr(ref, rec):

rec = rec / (jnp.sqrt(jnp.mean(jnp.abs(rec)**2)) + 1e-12)

ref = ref / (jnp.sqrt(jnp.mean(jnp.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = jnp.roll(rec, shift)

for angle in [0, jnp.pi]: # PAM4 只有 0 和 180 度翻转

temp_rec = r_sh * jnp.exp(-1j * angle)

phase_offset = jnp.angle(jnp.mean(temp_rec * jnp.conj(ref)))

temp_rec_final = temp_rec * jnp.exp(-1j * phase_offset)

mse = jnp.mean(jnp.abs(ref - temp_rec_final)**2)

snr = 10 * jnp.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def apply_chromatic_dispersion(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * jnp.pi * c) * (D * 1e-6)

f = jnp.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * jnp.pi * f

H = jnp.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = jnp.fft.fft(sig, axis=0)

return jnp.fft.ifft(sig_f * H[:, None], axis=0)

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / jnp.sqrt(2)) * ((1 + 2 / np.pi) * jnp.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * jnp.cos(np.pi / (4 * beta)))

else:

num = jnp.sin(np.pi * val * (1 - beta)) + 4 * beta * val * jnp.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / jnp.sqrt(jnp.sum(h**2))

# =========================================================================

# 【核心】：静态编译 JAX LMS 均衡器

# =========================================================================

@jax.jit(static_argnames=['taps', 'train_len'])

def pam4_lms_equalizer(rx, tx_train, const, taps=3, lr_da=5e-3, lr_dd=1e-3, train_len=5000):

n_syms = rx.shape[0]

pad = taps // 2

rx_pad = jnp.pad(rx, ((pad, pad), (0, 0)))

def step(W, i):

x = jax.lax.dynamic_slice(rx_pad, (i, 0), (taps, 2))

x = jnp.flip(x, axis=0)

y0 = jnp.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = jnp.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = jnp.array([y0, y1])

d_da = tx_train[i]

dist0 = jnp.abs(y0 - const)

dist1 = jnp.abs(y1 - const)

d_dd = jnp.array([const[jnp.argmin(dist0)], const[jnp.argmin(dist1)]])

is_train = i < train_len

d = jnp.where(is_train, d_da, d_dd)

lr = jnp.where(is_train, lr_da, lr_dd)

e = d - y

W = W.at[0, 0].add(lr * e[0] * jnp.conj(x[:, 0]))

W = W.at[0, 1].add(lr * e[0] * jnp.conj(x[:, 1]))

W = W.at[1, 0].add(lr * e[1] * jnp.conj(x[:, 0]))

W = W.at[1, 1].add(lr * e[1] * jnp.conj(x[:, 1]))

return W, y

W0 = jnp.zeros((2, 2, taps), dtype=jnp.complex64)

W0 = W0.at[0, 0, taps//2].set(1.0)

W0 = W0.at[1, 1, taps//2].set(1.0)

_, rx_eq = jax.lax.scan(step, W0, jnp.arange(n_syms))

return rx_eq

def get_ber(tx_aligned, rx_aligned, const):

"""

计算接收信号的误码率 (BER) 和符号误码率 (SER)

tx_aligned: 对齐后的理想发送符号 (1D array)

rx_aligned: 均衡并相位恢复后的接收符号 (1D array)

const: 标准星座图复数点坐标

"""

# 1. 找到发送信号和接收信号各自对应的最近星座点索引 (硬判决)

# 利用 JAX 的广播机制，瞬间计算所有点到所有星座点的欧氏距离

dist_tx = jnp.abs(tx_aligned[:, None] - const[None, :])

tx_idx = jnp.argmin(dist_tx, axis=-1)

dist_rx = jnp.abs(rx_aligned[:, None] - const[None, :])

rx_idx = jnp.argmin(dist_rx, axis=-1)

# 计算符号错误率 (SER)

ser_errors = jnp.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

# 2. 将星座点索引转换为二进制比特 (Bit Demapping)

M = len(const)

num_bits = int(np.log2(M)) # 例如 32QAM 就是 5 bits, 16QAM 就是 4 bits

# 利用位运算提取每一位比特

rx_bits = jnp.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = jnp.array([(tx_idx >> i) & 1 for i in range(num_bits)])

# 3. 计算误码率 (BER)

ber_errors = jnp.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

CPR,

ak,

apply_chromatic_dispersion,

get_ber,

get_safe_rrc_taps,

get_snr,

jax,

jnp,

np,

pam4_lms_equalizer,

plt,

sp_signal,

sym_map,

)

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="色散系数 (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="光纤长度 (km)")

lw_slider = mo.ui.slider(start=0, stop=100e3, step=5e3, value=10e3, label="激光器线宽 (Hz)")

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.01, value=0.04, label="噪声幅度")

mo.md(f"""

### 交互式物理损伤控制台 (PAM4 JAX 极速版)

| 物理参数 | 实时控制滑块 |

| :--- | :--- |

| **光纤色散系数** | {D_slider} |

| **光纤传输距离** | {L_slider} |

| **激光器线宽 (相位噪声)** | {lw_slider} |

| **信道 AWGN 噪声幅度** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(get_safe_rrc_taps, jax, jnp, np, sp_signal, sym_map):

num_syms = 32768

baud_rate = 32e9

const = jnp.asarray(sym_map.const('PAM4', norm=True))

_key = jax.random.PRNGKey(42)

_tx_indices = jax.random.randint(_key, (num_syms, 2), 0, 4)

tx_syms = const[_tx_indices]

rrc_taps_len = 65

h_rrc = get_safe_rrc_taps(sps=2, num_taps=rrc_taps_len, beta=0.1)

_tx_sig = sp_signal.upfirdn(np.array(h_rrc), np.array(tx_syms), up=2, axis=0)

delay = (rrc_taps_len - 1) // 2

tx_2sps = jnp.array(_tx_sig[delay : delay + num_syms * 2])

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(

D_slider,

L_slider,

apply_chromatic_dispersion,

baud_rate,

delay,

h_rrc,

jax,

jnp,

lw_slider,

np,

num_syms,

plt,

snr_slider,

sp_signal,

tx_2sps,

):

_theta = jnp.pi / 4 * 0.6

_J_rot = jnp.array([[jnp.cos(_theta), jnp.sin(_theta)],

[-jnp.sin(_theta), jnp.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_Ts = 1.0 / (baud_rate * 2)

_sigma = jnp.sqrt(2 * jnp.pi * lw_slider.value * _Ts)

_phase_steps = _sigma * jax.random.normal(jax.random.PRNGKey(99), (_rx_rot.shape[0], 1))

_rx_pn = _rx_rot * jnp.exp(1j * jnp.cumsum(_phase_steps, axis=0))

_rx_cd = apply_chromatic_dispersion(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

_noise = (jax.random.normal(jax.random.PRNGKey(123), _rx_cd.shape) +

1j * jax.random.normal(jax.random.PRNGKey(124), _rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

# 接收端静态 CDC 与 MF

_rx_cdc = apply_chromatic_dispersion(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(np.array(h_rrc), np.array(_rx_cdc), up=1, axis=0)

rx_mf_aligned = jnp.array(_rx_mf[delay : delay + num_syms * 2])

# 画图：PAM4 特有的 3个眼孔 眼图

fig_eye_pam4, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[10000:10000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'PAM4 Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye_pam4

return (rx_mf_aligned,)

@app.cell

def _(const, jnp, mo, pam4_lms_equalizer, rx_mf_aligned, tx_syms):

mo.md("**正在执行 JAX 加速的 DA/DD-LMS ...**")

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps = _rx_1sps / jnp.sqrt(jnp.mean(jnp.abs(_rx_1sps)**2, axis=0))

# 调用 JIT 编译的 LMS 均衡器

_rx_eq = pam4_lms_equalizer(_rx_1sps, tx_syms, const, taps=3)

start_idx = 6000

_rx_eq_cut_temp = _rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / jnp.sqrt(jnp.mean(jnp.abs(_rx_eq_cut_temp)**2, axis=0))

return rx_eq_cut, tx_final

@app.cell

def _(CPR, ak, jax, jnp, rx_eq_cut):

_rx_cpr_list = []

for _pol_idx in range(2):

_cpr_module = CPR(kernel=ak.cpr_4thpower_pll(mu=0.01))

_, _pol_cpr_out = jax.lax.scan(lambda c, y: c(y), _cpr_module, rx_eq_cut[:, _pol_idx])

_rx_cpr_list.append(_pol_cpr_out)

rx_cpr_all = jnp.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(const, get_ber, get_snr, jnp, mo, plt, rx_cpr_all, rx_eq_cut, tx_final):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = jnp.stack(_aligned_list, axis=1)

# 1. Assign the Markdown output to a variable

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

fig_diag = plt.figure(figsize=(15, 5))

# 1. LMS 后 (评估偏振是否解开，实轴是否收敛)

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-4000:, 0].real, rx_eq_cut[-4000:, 0].imag, s=2, alpha=0.3, color='tab:blue')

_ax1.set_title("1. After JAX LMS")

_ax1.axis('equal'); _ax1.grid(True, linestyle=':', alpha=0.6)

# 2. 最终 PAM4 星座图

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-4000:, 0].real, rx_aligned_all[-4000:, 0].imag, s=2, alpha=0.3, color='tab:green')

_ax2.set_title(f"2. Final DP-PAM4\nSNR: {float(snr_vals[0]):.2f} dB")

_ax2.axis('equal')

# 严格限制 Y 轴视角，凸显 PAM4 一维线状特性

_ax2.set_ylim([-0.5, 0.5])

_ax2.scatter(const.real, jnp.zeros_like(const.real), c='red', marker='x', s=40, alpha=0.9)

_ax2.grid(True, linestyle=':', alpha=0.6)

# 3. PLL 追踪轨迹

_phase_diff = jnp.unwrap(jnp.angle(rx_cpr_all[-1000:, 0] * jnp.conj(rx_eq_cut[-1000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1.5)

_ax3.set_title("3. PLL Phase Track Residual")

_ax3.set_xlabel("Symbols"); _ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

mo.vstack([performance_report, fig_diag])

return

if __name__ == "__main__":

app.run()

import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent DP-PAM4 Optical Communication System: Pure NumPy Interactive Comparison Version

This is a DP-PAM4 simulator built purely using `NumPy` and `SciPy`, without relying on any deep learning frameworks.

In this version, the core DA/DD-LMS equalizer is calculated symbol-by-symbol using a **native Python `for` loop**. Please feel free to scroll through the console below and experience the **real "lag" without JAX compilation acceleration**!

"""

)

return (mo,)

@app.cell

def _():

import time

import numpy as np

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

def get_snr_pam4_numpy(ref, rec):

rec = rec / (np.sqrt(np.mean(np.abs(rec)**2)) + 1e-12)

ref = ref / (np.sqrt(np.mean(np.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = np.roll(rec, shift)

for angle in [0, np.pi]:

temp_rec = r_sh * np.exp(-1j * angle)

phase_offset = np.angle(np.mean(temp_rec * np.conj(ref)))

temp_rec_final = temp_rec * np.exp(-1j * phase_offset)

mse = np.mean(np.abs(ref - temp_rec_final)**2)

snr = 10 * np.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / np.sqrt(2)) * ((1 + 2 / np.pi) * np.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * np.cos(np.pi / (4 * beta)))

else:

num = np.sin(np.pi * val * (1 - beta)) + 4 * beta * val * np.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / np.sqrt(np.sum(h**2))

def apply_cd_numpy(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * np.pi * c) * (D * 1e-6)

f = np.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * np.pi * f

H = np.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = np.fft.fft(sig, axis=0)

return np.fft.ifft(sig_f * H[:, np.newaxis], axis=0)

def pam4_lms_equalizer_numpy(rx, tx_train, const, taps=3, lr_da=5e-3, lr_dd=1e-3, train_len=5000):

n_syms = rx.shape[0]

pad = taps // 2

rx_pad = np.pad(rx, ((pad, pad), (0, 0)), mode='constant')

W = np.zeros((2, 2, taps), dtype=complex)

W[0, 0, taps//2] = 1.0

W[1, 1, taps//2] = 1.0

out = np.zeros_like(rx, dtype=complex)

for i in range(n_syms):

x = rx_pad[i : i+taps, :]

x = x[::-1, :]

y0 = np.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = np.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = np.array([y0, y1])

out[i] = y

if i < train_len:

d = tx_train[i]

lr = lr_da

else:

dist0 = np.abs(y0 - const)

dist1 = np.abs(y1 - const)

d = np.array([const[np.argmin(dist0)], const[np.argmin(dist1)]])

lr = lr_dd

e = d - y

W[0, 0] += lr * e[0] * np.conj(x[:, 0])

W[0, 1] += lr * e[0] * np.conj(x[:, 1])

W[1, 0] += lr * e[1] * np.conj(x[:, 0])

W[1, 1] += lr * e[1] * np.conj(x[:, 1])

return out

def cpr_4th_power_numpy(sig, mu=0.01):

n = len(sig)

out = np.zeros(n, dtype=complex)

phi = 0.0

for i in range(n):

out[i] = sig[i] * np.exp(-1j * phi)

err = np.angle(out[i]**4) / 4.0

phi += mu * err

return out

def get_ber(tx_aligned, rx_aligned, const):

"""完全使用 NumPy 实现的 BER 计算"""

dist_tx = np.abs(tx_aligned[:, None] - const[None, :])

tx_idx = np.argmin(dist_tx, axis=-1)

dist_rx = np.abs(rx_aligned[:, None] - const[None, :])

rx_idx = np.argmin(dist_rx, axis=-1)

ser_errors = np.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

M = len(const)

num_bits = int(np.log2(M))

rx_bits = np.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = np.array([(tx_idx >> i) & 1 for i in range(num_bits)])

ber_errors = np.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

apply_cd_numpy,

cpr_4th_power_numpy,

get_ber,

get_safe_rrc_taps,

get_snr_pam4_numpy,

np,

pam4_lms_equalizer_numpy,

plt,

sp_signal,

time,

)

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="色散系数 (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="光纤长度 (km)")

lw_slider = mo.ui.slider(start=0, stop=100e3, step=5e3, value=10e3, label="激光器线宽 (Hz)")

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.01, value=0.04, label="噪声幅度")

mo.md(f"""

### 交互式控制台 (NumPy)

| 物理参数 | 实时控制滑块 |

| :--- | :--- |

| **光纤色散系数** | {D_slider} |

| **光纤传输距离** | {L_slider} |

| **激光器线宽 (相位噪声)** | {lw_slider} |

| **信道 AWGN 噪声幅度** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(get_safe_rrc_taps, np, sp_signal):

num_syms = 32768

baud_rate = 32e9

_pam_levels = np.array([-3, -1, 1, 3], dtype=complex)

const = _pam_levels / np.sqrt(np.mean(np.abs(_pam_levels)**2))

np.random.seed(42)

_tx_indices = np.random.randint(0, 4, (num_syms, 2))

tx_syms = const[_tx_indices]

h_rrc = get_safe_rrc_taps(sps=2, num_taps=65, beta=0.1)

_tx_sig = sp_signal.upfirdn(h_rrc, tx_syms, up=2, axis=0)

delay = 32

tx_2sps = _tx_sig[delay : delay + num_syms * 2]

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(

D_slider,

L_slider,

apply_cd_numpy,

baud_rate,

delay,

h_rrc,

lw_slider,

np,

num_syms,

plt,

snr_slider,

sp_signal,

tx_2sps,

):

_theta = np.pi / 4 * 0.6

_J_rot = np.array([[np.cos(_theta), np.sin(_theta)], [-np.sin(_theta), np.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_sigma = np.sqrt(2 * np.pi * lw_slider.value * (1.0 / (baud_rate * 2)))

# 固定噪声种子以防止每次重绘时眼图乱跳

np.random.seed(99)

_phase_noise = np.exp(1j * np.cumsum(_sigma * np.random.standard_normal((_rx_rot.shape[0], 1)), axis=0))

_rx_pn = _rx_rot * _phase_noise

_rx_cd = apply_cd_numpy(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

np.random.seed(123)

_noise = (np.random.standard_normal(_rx_cd.shape) + 1j * np.random.standard_normal(_rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

# 接收端 CDC 与 MF

_rx_cdc = apply_cd_numpy(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(h_rrc, _rx_cdc, up=1, axis=0)

rx_mf_aligned = _rx_mf[delay : delay + num_syms * 2]

fig_eye_pam4_np, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[10000:10000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'PAM4 Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye_pam4_np

return (rx_mf_aligned,)

@app.cell

def _(const, mo, np, pam4_lms_equalizer_numpy, rx_mf_aligned, time, tx_syms):

_start_lms = time.time()

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps /= np.sqrt(np.mean(np.abs(_rx_1sps)**2, axis=0))

# 这将是一段漫长的 Python for 循环等待...

_rx_eq = pam4_lms_equalizer_numpy(_rx_1sps, tx_syms, const, taps=3, train_len=5000)

start_idx = 6000

_rx_eq_cut_temp = _rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / np.sqrt(np.mean(np.abs(_rx_eq_cut_temp)**2, axis=0))

_end_lms = time.time()

lms_time = _end_lms - _start_lms

mo.md(f"### LMS opration time: **{lms_time:.2f} s**")

return rx_eq_cut, tx_final

@app.cell

def _(cpr_4th_power_numpy, np, rx_eq_cut):

_rx_cpr_list = []

for _i in range(2):

_rx_cpr_list.append(cpr_4th_power_numpy(rx_eq_cut[:, _i], mu=0.01))

rx_cpr_all = np.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(

const,

get_ber,

get_snr_pam4_numpy,

mo,

np,

plt,

rx_cpr_all,

rx_eq_cut,

tx_final,

):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr_pam4_numpy(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = np.stack(_aligned_list, axis=1)

# 1. 把 Markdown 赋值给变量

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

fig_diag_np = plt.figure(figsize=(15, 5))

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-4000:, 0].real, rx_eq_cut[-4000:, 0].imag, s=1, alpha=0.5, color='tab:blue')

_ax1.set_title("1. After DA/DD-LMS (NumPy)")

_ax1.axis('equal'); _ax1.grid(True, linestyle=':', alpha=0.6)

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-4000:, 0].real, rx_aligned_all[-4000:, 0].imag, s=1, alpha=0.5, color='tab:green')

_ax2.set_title(f"2. Final DP-PAM4\nSNR: {snr_vals[0]:.2f} dB")

_ax2.axis('equal'); _ax2.set_ylim([-0.5, 0.5])

_ax2.scatter(const.real, np.zeros_like(const.real), c='red', marker='x', s=40, alpha=0.9)

_ax2.grid(True, linestyle=':', alpha=0.6)

_phase_diff = np.unwrap(np.angle(rx_cpr_all[-1000:, 0] * np.conj(rx_eq_cut[-1000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1)

_ax3.set_title("3. Phase Track Residual")

_ax3.set_xlabel("Symbols"); _ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

mo.vstack([performance_report, fig_diag_np])

return

if __name__ == "__main__":

app.run()

import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent Optical Communication DSP: Pure NumPy Comparison Version

This is a low-level DSP simulation implemented using pure `NumPy` and `SciPy`.

In this version, the CMA equalizer and CPR phase-locked loop are manually calculated step-by-step using **Python's native `for` loop**. You can experience the difference in time consumption compared to the JAX version when adjusting the slider below!

"""

)

return (mo,)

@app.cell

def _():

import time

import numpy as np

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

def get_snr_numpy(ref, rec):

rec = rec / (np.sqrt(np.mean(np.abs(rec)**2)) + 1e-12)

ref = ref / (np.sqrt(np.mean(np.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = np.roll(rec, shift)

for angle in [0, np.pi/2, np.pi, 3*np.pi/2]:

temp_rec = r_sh * np.exp(-1j * angle)

phase_offset = np.angle(np.mean(temp_rec * np.conj(ref)))

temp_rec_final = temp_rec * np.exp(-1j * phase_offset)

mse = np.mean(np.abs(ref - temp_rec_final)**2)

snr = 10 * np.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / np.sqrt(2)) * ((1 + 2 / np.pi) * np.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * np.cos(np.pi / (4 * beta)))

else:

num = np.sin(np.pi * val * (1 - beta)) + 4 * beta * val * np.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / np.sqrt(np.sum(h**2))

def apply_cd_numpy(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * np.pi * c) * (D * 1e-6)

f = np.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * np.pi * f

H = np.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = np.fft.fft(sig, axis=0)

return np.fft.ifft(sig_f * H[:, np.newaxis], axis=0)

def cma_equalizer_numpy(signal, taps=3, lr=2e-3, R2=1.32):

n_samples = signal.shape[0]

w = np.zeros((2, 2, taps), dtype=complex)

w[0, 0, taps//2] = 1.0

w[1, 1, taps//2] = 1.0

out = np.zeros_like(signal, dtype=complex)

for i in range(taps, n_samples):

x = signal[i-taps+1 : i+1, :].T

x = x[:, ::-1]

y = np.zeros(2, dtype=complex)

y[0] = np.sum(w[0, 0] * x[0] + w[0, 1] * x[1])

y[1] = np.sum(w[1, 0] * x[0] + w[1, 1] * x[1])

out[i] = y

for p in range(2):

err = R2 - np.abs(y[p])**2

w[p, 0] += lr * err * y[p] * np.conj(x[0])

w[p, 1] += lr * err * y[p] * np.conj(x[1])

return out

def cpr_4th_power_numpy(sig, mu=0.02):

n = len(sig)

out = np.zeros(n, dtype=complex)

phi = 0.0

for i in range(n):

out[i] = sig[i] * np.exp(-1j * phi)

err = np.angle(out[i]**4) / 4.0

phi += mu * err

return out

def get_ber(tx_aligned, rx_aligned, const):

"""

计算接收信号的误码率 (BER) 和符号误码率 (SER) (修复为纯 NumPy 版本)

"""

dist_tx = np.abs(tx_aligned[:, None] - const[None, :])

tx_idx = np.argmin(dist_tx, axis=-1)

dist_rx = np.abs(rx_aligned[:, None] - const[None, :])

rx_idx = np.argmin(dist_rx, axis=-1)

ser_errors = np.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

M = len(const)

num_bits = int(np.log2(M))

rx_bits = np.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = np.array([(tx_idx >> i) & 1 for i in range(num_bits)])

ber_errors = np.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

apply_cd_numpy,

cma_equalizer_numpy,

cpr_4th_power_numpy,

get_ber,

get_safe_rrc_taps,

get_snr_numpy,

np,

plt,

sp_signal,

)

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="色散系数 (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="光纤长度 (km)")

lw_slider = mo.ui.slider(start=0, stop=100e3, step=5e3, value=10e3, label="激光器线宽 (Hz)")

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.01, value=0.04, label="噪声幅度")

mo.md(f"""

### 交互式物理损伤控制台 (NumPy)

| 物理参数 | 实时控制滑块 |

| :--- | :--- |

| **光纤色散系数** | {D_slider} |

| **光纤传输距离** | {L_slider} |

| **激光器线宽 (相位噪声)** | {lw_slider} |

| **信道 AWGN 噪声幅度** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(get_safe_rrc_taps, np, sp_signal):

num_syms = 32768

baud_rate = 32e9

_points = np.array([-3, -1, 1, 3])

const = np.array([complex(i, j) for i in _points for j in _points])

const /= np.sqrt(np.mean(np.abs(const)**2))

np.random.seed(42)

_tx_indices = np.random.randint(0, 16, (num_syms, 2))

tx_syms = const[_tx_indices]

h_rrc = get_safe_rrc_taps(sps=2, num_taps=65, beta=0.1)

_tx_sig = sp_signal.upfirdn(h_rrc, tx_syms, up=2, axis=0)

delay = 32

tx_2sps = _tx_sig[delay : delay + num_syms * 2]

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(

D_slider,

L_slider,

apply_cd_numpy,

baud_rate,

delay,

h_rrc,

lw_slider,

np,

num_syms,

plt,

snr_slider,

sp_signal,

tx_2sps,

):

_theta = np.pi / 4 * 0.6

_J_rot = np.array([[np.cos(_theta), np.sin(_theta)], [-np.sin(_theta), np.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_sigma = np.sqrt(2 * np.pi * lw_slider.value * (1.0 / (baud_rate * 2)))

_phase_noise = np.exp(1j * np.cumsum(_sigma * np.random.standard_normal((_rx_rot.shape[0], 1)), axis=0))

_rx_pn = _rx_rot * _phase_noise

_rx_cd = apply_cd_numpy(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

_noise = (np.random.standard_normal(_rx_cd.shape) + 1j * np.random.standard_normal(_rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

# 接收端静态 CDC 与 MF

_rx_cdc = apply_cd_numpy(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(h_rrc, _rx_cdc, up=1, axis=0)

rx_mf_aligned = _rx_mf[delay : delay + num_syms * 2]

fig_eye, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[10000:10000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye

return (rx_mf_aligned,)

@app.cell

def _(cma_equalizer_numpy, mo, np, rx_mf_aligned, tx_syms):

mo.md("**正在执行 NumPy 版 CMA 均衡...**")

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps /= np.sqrt(np.mean(np.abs(_rx_1sps)**2, axis=0))

# 跑两次以确保收敛

_rx_eq_p1 = cma_equalizer_numpy(_rx_1sps, taps=3, lr=2e-3)

_rx_eq_p2 = cma_equalizer_numpy(_rx_eq_p1, taps=3, lr=1e-3)

start_idx = 3000

_rx_eq_cut_temp = _rx_eq_p2[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / np.sqrt(np.mean(np.abs(_rx_eq_cut_temp)**2, axis=0))

return rx_eq_cut, tx_final

@app.cell

def _(cpr_4th_power_numpy, np, rx_eq_cut):

_rx_cpr_list = []

for _i in range(2):

_cpr_out = cpr_4th_power_numpy(rx_eq_cut[:, _i], mu=0.02)

_rx_cpr_list.append(_cpr_out)

rx_cpr_all = np.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(

const,

get_ber,

get_snr_numpy,

mo,

np,

plt,

rx_cpr_all,

rx_eq_cut,

tx_final,

):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr_numpy(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = np.stack(_aligned_list, axis=1)

# Assign the Markdown output to a variable

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

# Generate diagnostic plot figure

fig_diag = plt.figure(figsize=(15, 5))

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-4000:, 0].real, rx_eq_cut[-4000:, 0].imag, s=1, alpha=0.5, color='tab:blue')

_ax1.set_title("1. After CMA (NumPy)")

_ax1.axis('equal')

_ax1.grid(True, linestyle=':', alpha=0.6)

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-4000:, 0].real, rx_aligned_all[-4000:, 0].imag, s=1, alpha=0.5, color='tab:green')

_ax2.set_title(f"2. Final Constellation\nSNR: {snr_vals[0]:.2f} dB")

_ax2.axis('equal')

_ax2.scatter(const.real, const.imag, c='red', marker='x', s=20, alpha=0.7)

_ax2.grid(True, linestyle=':', alpha=0.6)

_phase_diff = np.unwrap(np.angle(rx_cpr_all[-1000:, 0] * np.conj(rx_eq_cut[-1000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1)

_ax3.set_title("3. Phase Track")

_ax3.set_xlabel("Symbols")

_ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

# 将渲染内容赋给变量，并在 marimo 的规范下返回

mo.vstack([performance_report, mo.as_html(fig_diag)])

return

if __name__ == "__main__":

app.run()

#32qam

import os

import time

import jax

import jax.numpy as jnp

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

import numpy as np

# =========================================================================

# 路径配置

# =========================================================================

BASE_DIR = os.path.dirname(os.path.abspath(__file__))

FIGURE_PATH = os.path.join(BASE_DIR, 'results_32qam', 'figure')

if not os.path.exists(FIGURE_PATH):

os.makedirs(FIGURE_PATH)

# =========================================================================

# 核心评价与绘图函数

# =========================================================================

def get_snr(ref, rec):

rec = rec / (jnp.sqrt(jnp.mean(jnp.abs(rec)**2)) + 1e-12)

ref = ref / (jnp.sqrt(jnp.mean(jnp.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = jnp.roll(rec, shift)

# 32QAM 依然具有 90 度旋转对称性

for angle in [0, jnp.pi/2, jnp.pi, 3*jnp.pi/2]:

temp_rec = r_sh * jnp.exp(-1j * angle)

phase_offset = jnp.angle(jnp.mean(temp_rec * jnp.conj(ref)))

temp_rec_final = temp_rec * jnp.exp(-1j * phase_offset)

mse = jnp.mean(jnp.abs(ref - temp_rec_final)**2)

snr = 10 * jnp.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def plot_psd(sig, fs, name='Signal PSD', filename='psd_analysis.png'):

plt.figure(figsize=(10, 5))

for i in range(sig.shape[1]):

f, Pxx_den = sp_signal.welch(np.array(sig[:, i]), fs, nperseg=1024)

plt.semilogy(f / 1e9, Pxx_den, label=f'Pol {i}')

plt.title(name)

plt.xlabel('Frequency (GHz)')

plt.ylabel('PSD (V**2/Hz)')

plt.grid(True, which='both', linestyle='--', alpha=0.5)

plt.legend()

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, filename), dpi=300)

plt.close()

def plot_eye(sig, sps=2, name='Eye Diagram', amplitude_limit=2.0, filename='eye_diagram.png'):

plt.figure(figsize=(12, 5))

num_points = int(2 * sps) + 1

num_traces = 800

for i in range(2):

ax = plt.subplot(1, 2, i+1)

start = 20000 if sig.shape[0] > 20000 else 0

data = np.array(sig[start:, i].real)

traces = []

for j in range(0, min(len(data) - num_points, num_traces * sps), sps):

traces.append(data[j : j + num_points])

if len(traces) > 0:

t = np.linspace(0, 2, num_points)

for trace in traces:

ax.plot(t, trace, 'b-', alpha=0.05, linewidth=0.5)

ax.axvline(x=1.0, color='r', linestyle='--', alpha=0.3, linewidth=1)

ax.set_xlim([0, 2])

ax.set_ylim([-amplitude_limit, amplitude_limit])

ax.set_title(f'{name} - Pol {i}')

ax.set_xlabel('Time (Symbol Period)')

ax.set_ylabel('Amplitude')

ax.grid(True, linestyle='--', alpha=0.5)

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, filename), dpi=300)

plt.close()

# =========================================================================

# 物理损伤模块

# =========================================================================

def apply_chromatic_dispersion(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * jnp.pi * c) * (D * 1e-6)

f = jnp.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * jnp.pi * f

H = jnp.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = jnp.fft.fft(sig, axis=0)

return jnp.fft.ifft(sig_f * H[:, None], axis=0)

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / np.sqrt(2)) * ((1 + 2 / np.pi) * np.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * np.cos(np.pi / (4 * beta)))

else:

num = np.sin(np.pi * val * (1 - beta)) + 4 * beta * val * np.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / np.sqrt(np.sum(h**2))

# =========================================================================

# JAX 极速自适应 DSP 算法核心 (专为 32QAM 打造)

# =========================================================================

@jax.jit(static_argnames=['taps', 'train_len'])

def qam32_lms_equalizer(rx, tx_train, const, taps=5, lr_da=2e-3, lr_dd=5e-4, train_len=10000):

n_syms = rx.shape[0]

pad = taps // 2

rx_pad = jnp.pad(rx, ((pad, pad), (0, 0)))

def step(W, i):

x = jax.lax.dynamic_slice(rx_pad, (i, 0), (taps, 2))

x = jnp.flip(x, axis=0)

y0 = jnp.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = jnp.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = jnp.array([y0, y1])

d_da = tx_train[i]

# 32QAM 密集二维复平面距离判决

dist0 = jnp.abs(y0 - const)

dist1 = jnp.abs(y1 - const)

d_dd = jnp.array([const[jnp.argmin(dist0)], const[jnp.argmin(dist1)]])

is_train = i < train_len

d = jnp.where(is_train, d_da, d_dd)

lr = jnp.where(is_train, lr_da, lr_dd)

e = d - y

W = W.at[0, 0].add(lr * e[0] * jnp.conj(x[:, 0]))

W = W.at[0, 1].add(lr * e[0] * jnp.conj(x[:, 1]))

W = W.at[1, 0].add(lr * e[1] * jnp.conj(x[:, 0]))

W = W.at[1, 1].add(lr * e[1] * jnp.conj(x[:, 1]))

return W, y

W0 = jnp.zeros((2, 2, taps), dtype=jnp.complex64)

W0 = W0.at[0, 0, taps//2].set(1.0)

W0 = W0.at[1, 1, taps//2].set(1.0)

_, rx_eq = jax.lax.scan(step, W0, jnp.arange(n_syms))

return rx_eq

@jax.jit

def dd_pll(rx_syms, const, mu=0.015):

"""判决引导锁相环 (Decision-Directed PLL)，专门对付十字形 32QAM"""

def step(phi, y_in):

# 补偿相位

y_rot = y_in * jnp.exp(-1j * phi)

# 硬判决找到最近的理想星座点

idx = jnp.argmin(jnp.abs(y_rot - const))

d = const[idx]

# 计算当前旋转点与理想点之间的相位差

err = jnp.angle(y_rot * jnp.conj(d))

# 环路滤波更新

phi_next = phi + mu * err

return phi_next, y_rot

_, rx_out = jax.lax.scan(step, 0.0, rx_syms)

return rx_out

# =========================================================================

# 主仿真逻辑

# =========================================================================

def main():

start_time = time.time()

print("Step 1: 32QAM Signal Generation & Pulse Shaping...")

# 【暴增数据量】：131072 个符号！

num_syms = 131072

baud_rate = 32e9

# 手动精细生成 32QAM 十字形星座图 (Cross-QAM)

grid = np.array([-5, -3, -1, 1, 3, 5])

X, Y = np.meshgrid(grid, grid)

c_raw = (X + 1j*Y).flatten()

# 剔除四个角落的点 (±5 ±5j)，剩下 36-4=32 个点

corners = [5+5j, 5-5j, -5+5j, -5-5j]

c_32qam = np.array([pt for pt in c_raw if pt not in corners])

const = jnp.array(c_32qam)

const = const / jnp.sqrt(jnp.mean(jnp.abs(const)**2)) # 严格归一化

key = jax.random.PRNGKey(42)

tx_indices = jax.random.randint(key, (num_syms, 2), 0, 32)

tx_syms = const[tx_indices]

rrc_taps_len = 65

h_rrc = get_safe_rrc_taps(sps=2, num_taps=rrc_taps_len, beta=0.1)

tx_sig = sp_signal.upfirdn(np.array(h_rrc), np.array(tx_syms), up=2, axis=0)

delay = (rrc_taps_len - 1) // 2

rx_2sps = jnp.array(tx_sig[delay : delay + num_syms * 2])

print("Step 2: Realistic Channel Impairments...")

theta = jnp.pi / 4 * 0.6

J_rot = jnp.array([[jnp.cos(theta), jnp.sin(theta)],

[-jnp.sin(theta), jnp.cos(theta)]])

rx_2sps = rx_2sps @ J_rot.T

lw = 10e3

Ts = 1.0 / (baud_rate * 2)

sigma = jnp.sqrt(2 * jnp.pi * lw * Ts)

phase_steps = sigma * jax.random.normal(jax.random.PRNGKey(99), (rx_2sps.shape[0], 1))

rx_2sps = rx_2sps * jnp.exp(1j * jnp.cumsum(phase_steps, axis=0))

rx_2sps = apply_chromatic_dispersion(rx_2sps, baud_rate, sps=2, D=17.0, L=80.0)

# 注意：32QAM 星座点极度密集，抗噪极差。我们将 AWGN 噪声设定为 0.02

noise = (jax.random.normal(jax.random.PRNGKey(123), rx_2sps.shape) +

1j * jax.random.normal(jax.random.PRNGKey(124), rx_2sps.shape)) * 0.02

rx_2sps += noise

print("Generating Pre-DSP Plots...")

plot_psd(rx_2sps, fs=baud_rate*2, name='PSD Before DSP', filename='01_psd_before_dsp.png')

plot_eye(rx_2sps, sps=2, name='32QAM Eye Before DSP', filename='02_eye_before_dsp.png')

print("Step 2.5: Static CDC & Rx Matched Filtering...")

rx_2sps = apply_chromatic_dispersion(rx_2sps, baud_rate, sps=2, D=17.0, L=-80.0)

rx_mf = sp_signal.upfirdn(np.array(h_rrc), np.array(rx_2sps), up=1, axis=0)

rx_2sps = jnp.array(rx_mf[delay : delay + num_syms * 2])

plot_eye(rx_2sps, sps=2, name='32QAM Eye After CDC & MF', filename='02.5_eye_after_cdc_mf.png')

print(f"Step 3: JAX LMS Equalization ({num_syms} symbols!)...")

rx_1sps = rx_2sps[::2]

rx_1sps = rx_1sps / jnp.sqrt(jnp.mean(jnp.abs(rx_1sps)**2, axis=0))

# Taps 增加到 5，加长训练序列到 10000 保证完美解开密集的偏振串扰

rx_eq = qam32_lms_equalizer(rx_1sps, tx_syms, const, taps=5, train_len=10000)

print("Step 4: Discarding convergence transient & Normalization...")

start_idx = 15000

rx_eq_cut = rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + rx_eq_cut.shape[0]]

rx_eq_cut = rx_eq_cut / jnp.sqrt(jnp.mean(jnp.abs(rx_eq_cut)**2, axis=0))

print("Step 5: Decision-Directed Carrier Phase Recovery (DD-PLL)...")

rx_cpr_list = []

for i in range(2):

pol_cpr_out = dd_pll(rx_eq_cut[:, i], const, mu=0.015)

rx_cpr_list.append(pol_cpr_out)

rx_cpr_all = jnp.stack(rx_cpr_list, axis=1)

print("Step 6: Performance Evaluation & Alignment...")

aligned_list = []

snr_vals = []

for i in range(2):

snr_v, aligned_sig = get_snr(tx_final[:, i], rx_cpr_all[:, i])

print(f"   Pol {i} Final SNR: {snr_v:.2f} dB")

aligned_list.append(aligned_sig)

snr_vals.append(snr_v)

rx_aligned_all = jnp.stack(aligned_list, axis=1)

print("Generating Final DSP Plots & Diagnostics...")

plt.figure(figsize=(15, 5))

ax1 = plt.subplot(1, 3, 1)

ax1.scatter(rx_eq_cut[-8000:, 0].real, rx_eq_cut[-8000:, 0].imag, s=1, alpha=0.3, color='tab:blue')

ax1.set_title("1. After JAX LMS (Ring Structure)")

ax1.axis('equal'); ax1.grid(True, linestyle=':', alpha=0.6)

ax2 = plt.subplot(1, 3, 2)

ax2.scatter(rx_aligned_all[-8000:, 0].real, rx_aligned_all[-8000:, 0].imag, s=1, alpha=0.3, color='tab:green')

ax2.set_title(f"2. Final 32QAM\nSNR: {float(snr_vals[0]):.2f} dB")

ax2.axis('equal')

ax2.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

ax2.grid(True, linestyle=':', alpha=0.6)

phase_diff = jnp.unwrap(jnp.angle(rx_cpr_all[-2000:, 0] * jnp.conj(rx_eq_cut[-2000:, 0])))

ax3 = plt.subplot(1, 3, 3)

ax3.plot(phase_diff, color='tab:orange', linewidth=1)

ax3.set_title("3. DD-PLL Phase Track")

ax3.set_xlabel("Symbols"); ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, '00_diagnostic_plot.png'), dpi=300)

plt.close()

plt.figure(figsize=(10, 5))

for i in range(2):

ax = plt.subplot(1, 2, i+1)

ax.scatter(rx_aligned_all[-8000:, i].real, rx_aligned_all[-8000:, i].imag, s=1, alpha=0.3)

ax.set_title(f'32QAM Constellation Pol {i}\nSNR: {float(snr_vals[i]):.2f} dB')

ax.axis('equal')

ax.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

ax.grid(True, linestyle=':', alpha=0.5)

plt.savefig(os.path.join(FIGURE_PATH, '03_constellations.png'), dpi=300)

plt.close()

plot_psd(rx_aligned_all, fs=baud_rate, name='PSD After DSP', filename='04_psd_after_dsp.png')

plot_eye(rx_aligned_all, sps=1, name='Eye After DSP (Recovered)', amplitude_limit=1.5, filename='05_eye_after_dsp.png')

end_time = time.time()

print("\n" + "="*55)

print(f"32QAM 极速仿真结束")

print(f"处理数据量: {num_syms} 个符号")

print(f"总共耗时: {end_time - start_time:.2f} 秒")

print(f"图像已保存至目录:\n   {FIGURE_PATH}\n")

print("本次生成的图像文件清单:")

if os.path.exists(FIGURE_PATH):

saved_files = [f for f in os.listdir(FIGURE_PATH) if f.endswith('.png')]

for idx, file_name in enumerate(sorted(saved_files), 1):

print(f"  [{idx}] {file_name}")

print("="*55 + "\n")

if __name__ == "__main__":

main()

import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent Optical Communication DSP Algorithm Practical Tutorial

Welcome to the JAX/Commplax optical communication simulation tutorial! In this interactive notebook, we will simulate a **32 GBaud 16QAM** signal, and after it has undergone physical damage, use a DSP algorithm to recover it.

"""

)

return (mo,)

@app.cell

def _():

import time

import jax

import jax.numpy as jnp

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

import numpy as np

from commplax import adaptive_kernel as ak

from commplax.equalizer import MIMOCell, CPR

from commplax import sym_map

def get_snr(ref, rec):

rec = rec / (jnp.sqrt(jnp.mean(jnp.abs(rec)**2)) + 1e-12)

ref = ref / (jnp.sqrt(jnp.mean(jnp.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = jnp.roll(rec, shift)

for angle in [0, jnp.pi/2, jnp.pi, 3*jnp.pi/2]:

temp_rec = r_sh * jnp.exp(-1j * angle)

phase_offset = jnp.angle(jnp.mean(temp_rec * jnp.conj(ref)))

temp_rec_final = temp_rec * jnp.exp(-1j * phase_offset)

mse = jnp.mean(jnp.abs(ref - temp_rec_final)**2)

snr = 10 * jnp.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def apply_chromatic_dispersion(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * jnp.pi * c) * (D * 1e-6)

f = jnp.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * jnp.pi * f

H = jnp.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = jnp.fft.fft(sig, axis=0)

return jnp.fft.ifft(sig_f * H[:, None], axis=0)

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / np.sqrt(2)) * ((1 + 2 / np.pi) * np.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * np.cos(np.pi / (4 * beta)))

else:

num = np.sin(np.pi * val * (1 - beta)) + 4 * beta * val * np.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / np.sqrt(np.sum(h**2))

def get_ber(tx_aligned, rx_aligned, const):

"""

计算接收信号的误码率 (BER) 和符号误码率 (SER)

tx_aligned: 对齐后的理想发送符号 (1D array)

rx_aligned: 均衡并相位恢复后的接收符号 (1D array)

const: 标准星座图复数点坐标

"""

# 1. 找到发送信号和接收信号各自对应的最近星座点索引 (硬判决)

# 利用 JAX 的广播机制，瞬间计算所有点到所有星座点的欧氏距离

dist_tx = jnp.abs(tx_aligned[:, None] - const[None, :])

tx_idx = jnp.argmin(dist_tx, axis=-1)

dist_rx = jnp.abs(rx_aligned[:, None] - const[None, :])

rx_idx = jnp.argmin(dist_rx, axis=-1)

# 计算符号错误率 (SER)

ser_errors = jnp.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

# 2. 将星座点索引转换为二进制比特 (Bit Demapping)

M = len(const)

num_bits = int(np.log2(M)) # 例如 32QAM 就是 5 bits, 16QAM 就是 4 bits

# 利用位运算提取每一位比特

rx_bits = jnp.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = jnp.array([(tx_idx >> i) & 1 for i in range(num_bits)])

# 3. 计算误码率 (BER)

ber_errors = jnp.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

CPR,

MIMOCell,

ak,

apply_chromatic_dispersion,

get_ber,

get_safe_rrc_taps,

get_snr,

jax,

jnp,

np,

plt,

sp_signal,

sym_map,

)

@app.cell

def _(get_safe_rrc_taps, jax, jnp, np, sp_signal, sym_map):

num_syms = 32768

baud_rate = 32e9

const = jnp.asarray(sym_map.const('16QAM', norm=True))

_key = jax.random.PRNGKey(42)

_tx_indices = jax.random.randint(_key, (num_syms, 2), 0, 16)

tx_syms = const[_tx_indices]

rrc_taps_len = 65

h_rrc = get_safe_rrc_taps(sps=2, num_taps=rrc_taps_len, beta=0.1)

_tx_sig = sp_signal.upfirdn(h_rrc, np.array(tx_syms), up=2, axis=0)

delay = (rrc_taps_len - 1) // 2

tx_2sps = jnp.array(_tx_sig[delay : delay + num_syms * 2])

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="Dispersion (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="Fiber Length (km)")

lw_slider = mo.ui.slider(start=0, stop=100e3, step=5e3, value=10e3, label="Laser Linewidth (Hz)")

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.01, value=0.04, label="Noise Level")

# Display them in the marimo page using Markdown

mo.md(f"""

### Interactive Physical Impairments Console

Drag the sliders below, and the underlying fiber impairments, DSP algorithms, and the final constellations will be **recalculated and rendered in real-time**!

| Physical Parameters | Real-time Control Sliders |

| :--- | :--- |

| **Fiber Dispersion Coefficient** | {D_slider} |

| **Fiber Transmission Distance** | {L_slider} |

| **Laser Linewidth (Phase Noise)** | {lw_slider} |

| **Channel AWGN Noise Level** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(

D_slider,

L_slider,

apply_chromatic_dispersion,

baud_rate,

jax,

jnp,

lw_slider,

snr_slider,

tx_2sps,

):

_theta = jnp.pi / 4 * 0.6

_J_rot = jnp.array([[jnp.cos(_theta), jnp.sin(_theta)],

[-jnp.sin(_theta), jnp.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_lw = lw_slider.value

_Ts = 1.0 / (baud_rate * 2)

_sigma = jnp.sqrt(2 * jnp.pi * _lw * _Ts)

_phase_steps = _sigma * jax.random.normal(jax.random.PRNGKey(99), (_rx_rot.shape[0], 1))

_rx_pn = _rx_rot * jnp.exp(1j * jnp.cumsum(_phase_steps, axis=0))

_rx_cd = apply_chromatic_dispersion(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

_noise = (jax.random.normal(jax.random.PRNGKey(123), _rx_cd.shape) +

1j * jax.random.normal(jax.random.PRNGKey(124), _rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

return (rx_channel_out,)

@app.cell

def _(

D_slider,

L_slider,

apply_chromatic_dispersion,

baud_rate,

delay,

h_rrc,

jnp,

np,

num_syms,

plt,

rx_channel_out,

sp_signal,

):

_rx_cdc = apply_chromatic_dispersion(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(h_rrc, np.array(_rx_cdc), up=1, axis=0)

rx_mf_aligned = jnp.array(_rx_mf[delay : delay + num_syms * 2])

fig_eye, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[10000:10000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye

return (rx_mf_aligned,)

@app.cell

def _(MIMOCell, ak, jax, jnp, rx_mf_aligned, tx_syms):

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps = _rx_1sps / jnp.sqrt(jnp.mean(jnp.abs(_rx_1sps)**2, axis=0))

_kernel = ak.cma(lr=2e-3, R2=1.32)

_cma_eq = MIMOCell(num_taps=3, dims=2, kernel=_kernel, up=1, down=1)

def scan_step(eq_state, x):

return eq_state(x)

_cma_trained, _ = jax.lax.scan(scan_step, _cma_eq, _rx_1sps)

_, _rx_eq = jax.lax.scan(scan_step, _cma_trained, _rx_1sps)

start_idx = 3000

_rx_eq_cut_temp = _rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / jnp.sqrt(jnp.mean(jnp.abs(_rx_eq_cut_temp)**2, axis=0))

return rx_eq_cut, tx_final

@app.cell

def _(CPR, ak, jax, jnp, rx_eq_cut):

_rx_cpr_list = []

for _pol_idx in range(2):

_cpr_module = CPR(kernel=ak.cpr_4thpower_pll(mu=0.02))

_, _pol_cpr_out = jax.lax.scan(lambda c, y: c(y), _cpr_module, rx_eq_cut[:, _pol_idx])

_rx_cpr_list.append(_pol_cpr_out)

rx_cpr_all = jnp.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(const, get_ber, get_snr, jnp, mo, plt, rx_cpr_all, rx_eq_cut, tx_final):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = jnp.stack(_aligned_list, axis=1)

# 1. Assign the Markdown output to a variable

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

fig_diag = plt.figure(figsize=(15, 5))

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-4000:, 0].real, rx_eq_cut[-4000:, 0].imag, s=2, alpha=0.3, color='tab:blue')

_ax1.set_title("1. After CMA (Before CPR)")

_ax1.axis('equal'); _ax1.grid(True, linestyle=':')

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-4000:, 0].real, rx_aligned_all[-4000:, 0].imag, s=2, alpha=0.3, color='tab:green')

_ax2.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.8)

_ax2.set_title(f"2. Final Constellation\nSNR: {float(snr_vals[0]):.2f} dB")

_ax2.axis('equal'); _ax2.grid(True, linestyle=':')

_phase_diff = jnp.unwrap(jnp.angle(rx_cpr_all[-1000:, 0] * jnp.conj(rx_eq_cut[-1000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1.5)

_ax3.set_title("3. PLL Phase Track")

_ax3.set_xlabel("Symbols"); _ax3.grid(True, linestyle=':')

plt.tight_layout()

# 2. Use mo.vstack as the final expression to render both objects vertically

mo.vstack([performance_report, fig_diag])

return

if __name__ == "__main__":

app.run()

import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent 32QAM Optical Communication System with JAX High-Speed ​​DSP Engine

32QAM has a very special **"Cross" constellation diagram**.

To cope with its dense decision boundaries, this tutorial deals with up to **131,072 symbols**. We will use a handwritten **JAX Decision-Guided Equalizer (DD-LMS)** and **Decision-Guided Phase-Locked Loop (DD-PLL)** instead of traditional blind algorithms.

"""

)

return (mo,)

@app.cell

def _():

import time

import jax

import jax.numpy as jnp

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

import numpy as np

def get_snr(ref, rec):

rec = rec / (jnp.sqrt(jnp.mean(jnp.abs(rec)**2)) + 1e-12)

ref = ref / (jnp.sqrt(jnp.mean(jnp.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = jnp.roll(rec, shift)

for angle in [0, jnp.pi/2, jnp.pi, 3*jnp.pi/2]:

temp_rec = r_sh * jnp.exp(-1j * angle)

phase_offset = jnp.angle(jnp.mean(temp_rec * jnp.conj(ref)))

temp_rec_final = temp_rec * jnp.exp(-1j * phase_offset)

mse = jnp.mean(jnp.abs(ref - temp_rec_final)**2)

snr = 10 * jnp.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def apply_chromatic_dispersion(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * jnp.pi * c) * (D * 1e-6)

f = jnp.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * jnp.pi * f

H = jnp.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = jnp.fft.fft(sig, axis=0)

return jnp.fft.ifft(sig_f * H[:, None], axis=0)

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / jnp.sqrt(2)) * ((1 + 2 / np.pi) * jnp.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * jnp.cos(np.pi / (4 * beta)))

else:

num = jnp.sin(np.pi * val * (1 - beta)) + 4 * beta * val * jnp.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / jnp.sqrt(jnp.sum(h**2))

@jax.jit(static_argnames=['taps', 'train_len'])

def qam32_lms_equalizer(rx, tx_train, const, taps=5, lr_da=2e-3, lr_dd=5e-4, train_len=10000):

n_syms = rx.shape[0]

pad = taps // 2

rx_pad = jnp.pad(rx, ((pad, pad), (0, 0)))

def step(W, i):

x = jax.lax.dynamic_slice(rx_pad, (i, 0), (taps, 2))

x = jnp.flip(x, axis=0)

y0 = jnp.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = jnp.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = jnp.array([y0, y1])

d_da = tx_train[i]

# 32QAM 密集二维复平面距离判决

dist0 = jnp.abs(y0 - const)

dist1 = jnp.abs(y1 - const)

d_dd = jnp.array([const[jnp.argmin(dist0)], const[jnp.argmin(dist1)]])

is_train = i < train_len

d = jnp.where(is_train, d_da, d_dd)

lr = jnp.where(is_train, lr_da, lr_dd)

e = d - y

W = W.at[0, 0].add(lr * e[0] * jnp.conj(x[:, 0]))

W = W.at[0, 1].add(lr * e[0] * jnp.conj(x[:, 1]))

W = W.at[1, 0].add(lr * e[1] * jnp.conj(x[:, 0]))

W = W.at[1, 1].add(lr * e[1] * jnp.conj(x[:, 1]))

return W, y

W0 = jnp.zeros((2, 2, taps), dtype=jnp.complex64)

W0 = W0.at[0, 0, taps//2].set(1.0)

W0 = W0.at[1, 1, taps//2].set(1.0)

_, rx_eq = jax.lax.scan(step, W0, jnp.arange(n_syms))

return rx_eq

@jax.jit

def dd_pll(rx_syms, const, mu=0.015):

"""判决引导锁相环 (Decision-Directed PLL)"""

def step(phi, y_in):

y_rot = y_in * jnp.exp(-1j * phi)

idx = jnp.argmin(jnp.abs(y_rot - const))

d = const[idx]

err = jnp.angle(y_rot * jnp.conj(d))

phi_next = phi + mu * err

return phi_next, y_rot

_, rx_out = jax.lax.scan(step, 0.0, rx_syms)

return rx_out

def get_ber(tx_aligned, rx_aligned, const):

"""

计算接收信号的误码率 (BER) 和符号误码率 (SER)

tx_aligned: 对齐后的理想发送符号 (1D array)

rx_aligned: 均衡并相位恢复后的接收符号 (1D array)

const: 标准星座图复数点坐标

"""

# 1. 找到发送信号和接收信号各自对应的最近星座点索引 (硬判决)

# 利用 JAX 的广播机制，瞬间计算所有点到所有星座点的欧氏距离

dist_tx = jnp.abs(tx_aligned[:, None] - const[None, :])

tx_idx = jnp.argmin(dist_tx, axis=-1)

dist_rx = jnp.abs(rx_aligned[:, None] - const[None, :])

rx_idx = jnp.argmin(dist_rx, axis=-1)

# 计算符号错误率 (SER)

ser_errors = jnp.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

# 2. 将星座点索引转换为二进制比特 (Bit Demapping)

M = len(const)

num_bits = int(np.log2(M)) # 例如 32QAM 就是 5 bits, 16QAM 就是 4 bits

# 利用位运算提取每一位比特

rx_bits = jnp.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = jnp.array([(tx_idx >> i) & 1 for i in range(num_bits)])

# 3. 计算误码率 (BER)

ber_errors = jnp.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

apply_chromatic_dispersion,

dd_pll,

get_ber,

get_safe_rrc_taps,

get_snr,

jax,

jnp,

np,

plt,

qam32_lms_equalizer,

sp_signal,

)

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="色散系数 (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="光纤长度 (km)")

lw_slider = mo.ui.slider(start=0, stop=50e3, step=2e3, value=10e3, label="激光器线宽 (Hz)")

# 注意：32QAM抗噪极差，所以我们把噪声滑块的范围缩小了

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.005, value=0.02, label="噪声幅度")

mo.md(f"""

### 交互式控制台 (32QAM 极速版)

| 物理参数 | 实时控制滑块 |

| :--- | :--- |

| **光纤色散系数** | {D_slider} |

| **光纤传输距离** | {L_slider} |

| **激光器线宽 (相位噪声)** | {lw_slider} |

| **信道 AWGN 噪声幅度** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(get_safe_rrc_taps, jax, jnp, np, sp_signal):

num_syms = 131072

baud_rate = 32e9

# 生成 32QAM 十字形星座图

_grid = np.array([-5, -3, -1, 1, 3, 5])

_X, _Y = np.meshgrid(_grid, _grid)

_c_raw = (_X + 1j*_Y).flatten()

_corners = [5+5j, 5-5j, -5+5j, -5-5j]

_c_32qam = np.array([pt for pt in _c_raw if pt not in _corners])

const = jnp.array(_c_32qam)

const = const / jnp.sqrt(jnp.mean(jnp.abs(const)**2))

_key = jax.random.PRNGKey(42)

_tx_indices = jax.random.randint(_key, (num_syms, 2), 0, 32)

tx_syms = const[_tx_indices]

rrc_taps_len = 65

h_rrc = get_safe_rrc_taps(sps=2, num_taps=rrc_taps_len, beta=0.1)

_tx_sig = sp_signal.upfirdn(np.array(h_rrc), np.array(tx_syms), up=2, axis=0)

delay = (rrc_taps_len - 1) // 2

tx_2sps = jnp.array(_tx_sig[delay : delay + num_syms * 2])

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(

D_slider,

L_slider,

apply_chromatic_dispersion,

baud_rate,

delay,

h_rrc,

jax,

jnp,

lw_slider,

np,

num_syms,

plt,

snr_slider,

sp_signal,

tx_2sps,

):

_theta = jnp.pi / 4 * 0.6

_J_rot = jnp.array([[jnp.cos(_theta), jnp.sin(_theta)],

[-jnp.sin(_theta), jnp.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_Ts = 1.0 / (baud_rate * 2)

_sigma = jnp.sqrt(2 * jnp.pi * lw_slider.value * _Ts)

_phase_steps = _sigma * jax.random.normal(jax.random.PRNGKey(99), (_rx_rot.shape[0], 1))

_rx_pn = _rx_rot * jnp.exp(1j * jnp.cumsum(_phase_steps, axis=0))

_rx_cd = apply_chromatic_dispersion(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

_noise = (jax.random.normal(jax.random.PRNGKey(123), _rx_cd.shape) +

1j * jax.random.normal(jax.random.PRNGKey(124), _rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

# CDC 与 匹配滤波

_rx_cdc = apply_chromatic_dispersion(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(np.array(h_rrc), np.array(_rx_cdc), up=1, axis=0)

rx_mf_aligned = jnp.array(_rx_mf[delay : delay + num_syms * 2])

# 画眼图

fig_eye_32qam, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[20000:20000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'32QAM Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye_32qam

return (rx_mf_aligned,)

@app.cell

def _(const, jnp, mo, qam32_lms_equalizer, rx_mf_aligned, tx_syms):

mo.md("**正在执行 JAX 加速的 DD-LMS ...**")

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps = _rx_1sps / jnp.sqrt(jnp.mean(jnp.abs(_rx_1sps)**2, axis=0))

_rx_eq = qam32_lms_equalizer(_rx_1sps, tx_syms, const, taps=5, train_len=10000)

start_idx = 15000

_rx_eq_cut_temp = _rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / jnp.sqrt(jnp.mean(jnp.abs(_rx_eq_cut_temp)**2, axis=0))

return rx_eq_cut, tx_final

@app.cell

def _(const, dd_pll, jnp, rx_eq_cut):

_rx_cpr_list = []

for _pol_idx in range(2):

_pol_cpr_out = dd_pll(rx_eq_cut[:, _pol_idx], const, mu=0.015)

_rx_cpr_list.append(_pol_cpr_out)

rx_cpr_all = jnp.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(const, get_ber, get_snr, jnp, mo, plt, rx_cpr_all, rx_eq_cut, tx_final):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = jnp.stack(_aligned_list, axis=1)

# 1. Assign the Markdown output to a variable

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

fig_diag = plt.figure(figsize=(15, 5))

# 1. LMS 后 (呈现出多个同心圆环结构)

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-8000:, 0].real, rx_eq_cut[-8000:, 0].imag, s=1, alpha=0.3, color='tab:blue')

_ax1.set_title("1. After JAX LMS (Ring Structure)")

_ax1.axis('equal'); _ax1.grid(True, linestyle=':', alpha=0.6)

# 2. 最终的 32QAM 十字形星座图

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-8000:, 0].real, rx_aligned_all[-8000:, 0].imag, s=1, alpha=0.3, color='tab:green')

_ax2.set_title(f"2. Final 32QAM\nSNR: {float(snr_vals[0]):.2f} dB")

_ax2.axis('equal')

_ax2.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

_ax2.grid(True, linestyle=':', alpha=0.6)

# 3. PLL 追踪轨迹

_phase_diff = jnp.unwrap(jnp.angle(rx_cpr_all[-2000:, 0] * jnp.conj(rx_eq_cut[-2000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1)

_ax3.set_title("3. DD-PLL Phase Track")

_ax3.set_xlabel("Symbols"); _ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

mo.vstack([performance_report, fig_diag])

return

if __name__ == "__main__":

app.run()

import marimo

__generated_with = "0.21.1"

app = marimo.App(width="medium")

@app.cell

def _():

import marimo as mo

mo.md(

"""

# Coherent 32QAM Optical Communication System: NumPy Performance Baseline

This notebook presents a simulation of a 32-Quadrature Amplitude Modulation (32QAM) coherent optical communication system utilizing standard `NumPy` and `SciPy` libraries.

Due to the complex decision boundaries of the cross-constellation, the simulation processes 131,072 symbols. The Decision-Directed Least Mean Squares (DD-LMS) equalizer and Decision-Directed Phase-Locked Loop (DD-PLL) are implemented via native Python iteration, serving as a baseline for computational performance evaluation.

"""

)

return (mo,)

@app.cell

def _():

import time

import numpy as np

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

def get_snr_numpy(ref, rec):

"""Calculate Signal-to-Noise Ratio (SNR) with phase alignment."""

rec = rec / (np.sqrt(np.mean(np.abs(rec)**2)) + 1e-12)

ref = ref / (np.sqrt(np.mean(np.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = np.roll(rec, shift)

# 32QAM maintains 90-degree rotational symmetry

for angle in [0, np.pi/2, np.pi, 3*np.pi/2]:

temp_rec = r_sh * np.exp(-1j * angle)

phase_offset = np.angle(np.mean(temp_rec * np.conj(ref)))

temp_rec_final = temp_rec * np.exp(-1j * phase_offset)

mse = np.mean(np.abs(ref - temp_rec_final)**2)

snr = 10 * np.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

"""Generate Root Raised Cosine (RRC) filter taps safely."""

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / np.sqrt(2)) * ((1 + 2 / np.pi) * np.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * np.cos(np.pi / (4 * beta)))

else:

num = np.sin(np.pi * val * (1 - beta)) + 4 * beta * val * np.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / np.sqrt(np.sum(h**2))

def apply_cd_numpy(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

"""Apply Chromatic Dispersion in the frequency domain."""

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * np.pi * c) * (D * 1e-6)

f = np.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * np.pi * f

H = np.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = np.fft.fft(sig, axis=0)

return np.fft.ifft(sig_f * H[:, np.newaxis], axis=0)

def qam32_lms_equalizer_numpy(rx, tx_train, const, taps=5, lr_da=2e-3, lr_dd=5e-4, train_len=10000):

"""Data-Aided / Decision-Directed LMS Equalizer for 32QAM."""

n_syms = rx.shape[0]

pad = taps // 2

rx_pad = np.pad(rx, ((pad, pad), (0, 0)), mode='constant')

W = np.zeros((2, 2, taps), dtype=complex)

W[0, 0, taps//2] = 1.0

W[1, 1, taps//2] = 1.0

out = np.zeros_like(rx, dtype=complex)

# Iterative symbol-by-symbol processing

for i in range(n_syms):

x = rx_pad[i : i+taps, :]

x = x[::-1, :]

y0 = np.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = np.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = np.array([y0, y1])

out[i] = y

if i < train_len:

d = tx_train[i]

lr = lr_da

else:

# Minimum Euclidean distance decision for 32QAM

dist0 = np.abs(y0 - const)

dist1 = np.abs(y1 - const)

d = np.array([const[np.argmin(dist0)], const[np.argmin(dist1)]])

lr = lr_dd

e = d - y

W[0, 0] += lr * e[0] * np.conj(x[:, 0])

W[0, 1] += lr * e[0] * np.conj(x[:, 1])

W[1, 0] += lr * e[1] * np.conj(x[:, 0])

W[1, 1] += lr * e[1] * np.conj(x[:, 1])

return out

def dd_pll_numpy(rx_syms, const, mu=0.015):

"""Decision-Directed Phase-Locked Loop (DD-PLL)."""

n = len(rx_syms)

out = np.zeros(n, dtype=complex)

phi = 0.0

for i in range(n):

y_rot = rx_syms[i] * np.exp(-1j * phi)

idx = np.argmin(np.abs(y_rot - const))

d = const[idx]

err = np.angle(y_rot * np.conj(d))

phi += mu * err

out[i] = y_rot

return out

def get_ber(tx_aligned, rx_aligned, const):

"""

纯 NumPy 版本的 BER 和 SER 计算 (修正版)

"""

dist_tx = np.abs(tx_aligned[:, None] - const[None, :])

tx_idx = np.argmin(dist_tx, axis=-1)

dist_rx = np.abs(rx_aligned[:, None] - const[None, :])

rx_idx = np.argmin(dist_rx, axis=-1)

ser_errors = np.sum(tx_idx != rx_idx)

total_syms = tx_idx.shape[0]

ser = ser_errors / total_syms

M = len(const)

num_bits = int(np.log2(M))

rx_bits = np.array([(rx_idx >> i) & 1 for i in range(num_bits)])

tx_bits = np.array([(tx_idx >> i) & 1 for i in range(num_bits)])

ber_errors = np.sum(rx_bits != tx_bits)

total_bits = total_syms * num_bits

ber = ber_errors / total_bits

return float(ber), int(ber_errors), total_bits, float(ser)

return (

apply_cd_numpy,

dd_pll_numpy,

get_ber,

get_safe_rrc_taps,

get_snr_numpy,

np,

plt,

qam32_lms_equalizer_numpy,

sp_signal,

time,

)

@app.cell

def _(mo):

D_slider = mo.ui.slider(start=0, stop=20, step=0.5, value=17.0, label="Dispersion Coefficient (ps/nm/km)")

L_slider = mo.ui.slider(start=0, stop=200, step=10, value=80.0, label="Fiber Length (km)")

lw_slider = mo.ui.slider(start=0, stop=50e3, step=2e3, value=10e3, label="Laser Linewidth (Hz)")

snr_slider = mo.ui.slider(start=0.005, stop=0.05, step=0.005, value=0.02, label="Noise Amplitude")

mo.md(f"""

### Interactive Physical Impairment Parameters

*Note: Modifying parameters triggers standard Python iteration, which may result in notable computational delays. Please observe the execution status indicator.*

| Parameter | Control |

| :--- | :--- |

| **Fiber Dispersion Coefficient** | {D_slider} |

| **Fiber Transmission Distance** | {L_slider} |

| **Laser Linewidth (Phase Noise)** | {lw_slider} |

| **Channel AWGN Noise Amplitude** | {snr_slider} |

""")

return D_slider, L_slider, lw_slider, snr_slider

@app.cell

def _(get_safe_rrc_taps, np, sp_signal):

num_syms = 131072

baud_rate = 32e9

# Manually generate 32QAM cross-constellation

_grid = np.array([-5, -3, -1, 1, 3, 5])

_X, _Y = np.meshgrid(_grid, _grid)

_c_raw = (_X + 1j*_Y).flatten()

_corners = [5+5j, 5-5j, -5+5j, -5-5j]

_c_32qam = np.array([pt for pt in _c_raw if pt not in _corners])

const = np.array(_c_32qam, dtype=complex)

const = const / np.sqrt(np.mean(np.abs(const)**2))

np.random.seed(42)

_tx_indices = np.random.randint(0, 32, (num_syms, 2))

tx_syms = const[_tx_indices]

h_rrc = get_safe_rrc_taps(sps=2, num_taps=65, beta=0.1)

_tx_sig = sp_signal.upfirdn(h_rrc, tx_syms, up=2, axis=0)

delay = 32

tx_2sps = _tx_sig[delay : delay + num_syms * 2]

return baud_rate, const, delay, h_rrc, num_syms, tx_2sps, tx_syms

@app.cell

def _(

D_slider,

L_slider,

apply_cd_numpy,

baud_rate,

delay,

h_rrc,

lw_slider,

np,

num_syms,

plt,

snr_slider,

sp_signal,

tx_2sps,

):

_theta = np.pi / 4 * 0.6

_J_rot = np.array([[np.cos(_theta), np.sin(_theta)], [-np.sin(_theta), np.cos(_theta)]])

_rx_rot = tx_2sps @ _J_rot.T

_sigma = np.sqrt(2 * np.pi * lw_slider.value * (1.0 / (baud_rate * 2)))

np.random.seed(99)

_phase_noise = np.exp(1j * np.cumsum(_sigma * np.random.standard_normal((_rx_rot.shape[0], 1)), axis=0))

_rx_pn = _rx_rot * _phase_noise

_rx_cd = apply_cd_numpy(_rx_pn, baud_rate, sps=2, D=D_slider.value, L=L_slider.value)

np.random.seed(123)

_noise = (np.random.standard_normal(_rx_cd.shape) + 1j * np.random.standard_normal(_rx_cd.shape)) * snr_slider.value

rx_channel_out = _rx_cd + _noise

# Static Chromatic Dispersion Compensation and Matched Filtering

_rx_cdc = apply_cd_numpy(rx_channel_out, baud_rate, sps=2, D=D_slider.value, L=-L_slider.value)

_rx_mf = sp_signal.upfirdn(h_rrc, _rx_cdc, up=1, axis=0)

rx_mf_aligned = _rx_mf[delay : delay + num_syms * 2]

fig_eye_32qam_np, _ax_eye = plt.subplots(1, 2, figsize=(10, 4))

for _i in range(2):

_traces = rx_mf_aligned[20000:20000+400*2, _i].real.reshape(-1, 5)

_ax_eye[_i].plot(np.linspace(0, 2, 5), _traces.T, 'b-', alpha=0.1)

_ax_eye[_i].set_title(f'32QAM Eye After CDC & MF - Pol {_i}')

_ax_eye[_i].grid(True, linestyle=':', alpha=0.5)

fig_eye_32qam_np

return (rx_mf_aligned,)

@app.cell

def _(

const,

mo,

np,

num_syms,

qam32_lms_equalizer_numpy,

rx_mf_aligned,

time,

tx_syms,

):

_start_lms = time.time()

_rx_1sps = rx_mf_aligned[::2]

_rx_1sps /= np.sqrt(np.mean(np.abs(_rx_1sps)**2, axis=0))

_rx_eq = qam32_lms_equalizer_numpy(_rx_1sps, tx_syms, const, taps=5, train_len=10000)

start_idx = 15000

_rx_eq_cut_temp = _rx_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + _rx_eq_cut_temp.shape[0]]

rx_eq_cut = _rx_eq_cut_temp / np.sqrt(np.mean(np.abs(_rx_eq_cut_temp)**2, axis=0))

_end_lms = time.time()

lms_time = _end_lms - _start_lms

mo.md(f"### Equalization Status\nExecution time for NumPy DD-LMS over {num_syms} symbols: **{lms_time:.2f} seconds**.")

return rx_eq_cut, tx_final

@app.cell

def _(const, dd_pll_numpy, np, rx_eq_cut):

_rx_cpr_list = []

for _i in range(2):

_rx_cpr_list.append(dd_pll_numpy(rx_eq_cut[:, _i], const, mu=0.015))

rx_cpr_all = np.stack(_rx_cpr_list, axis=1)

return (rx_cpr_all,)

@app.cell

def _(

const,

get_ber,

get_snr_numpy,

mo,

np,

plt,

rx_cpr_all,

rx_eq_cut,

tx_final,

):

_aligned_list = []

snr_vals = []

ber_vals = []

error_counts = []

for _eval_idx in range(2):

# Phase and time alignment via SNR calculation

_snr_v, _aligned_sig = get_snr_numpy(tx_final[:, _eval_idx], rx_cpr_all[:, _eval_idx])

# Calculate Bit Error Rate (BER) based on the aligned signals

_ber, _err_count, _total_bits, _ser = get_ber(tx_final[:, _eval_idx], _aligned_sig, const)

_aligned_list.append(_aligned_sig)

snr_vals.append(_snr_v)

ber_vals.append(_ber)

error_counts.append(_err_count)

rx_aligned_all = np.stack(_aligned_list, axis=1)

# 1. Assign the Markdown output to a variable

performance_report = mo.md(f"""

### Final System Performance Report

| Polarization | SNR | BER | Error Count | Total Bits Evaluated |

| :---: | :---: | :---: | :---: | :---: |

| **Pol 0 (X-Pol)** | **{snr_vals[0]:.2f} dB** | **{ber_vals[0]:.2e}** | {error_counts[0]} | {_total_bits} |

| **Pol 1 (Y-Pol)** | **{snr_vals[1]:.2f} dB** | **{ber_vals[1]:.2e}** | {error_counts[1]} | {_total_bits} |

*Note: Modern coherent optical communication systems typically require a Pre-FEC (Forward Error Correction) BER threshold of $2 \\times 10^{{-2}}$ for error-free transmission.*

""")

fig_diag_np = plt.figure(figsize=(15, 5))

_ax1 = plt.subplot(1, 3, 1)

_ax1.scatter(rx_eq_cut[-8000:, 0].real, rx_eq_cut[-8000:, 0].imag, s=1, alpha=0.3, color='tab:blue')

_ax1.set_title("1. After DA/DD-LMS")

_ax1.axis('equal'); _ax1.grid(True, linestyle=':', alpha=0.6)

_ax2 = plt.subplot(1, 3, 2)

_ax2.scatter(rx_aligned_all[-8000:, 0].real, rx_aligned_all[-8000:, 0].imag, s=1, alpha=0.3, color='tab:green')

_ax2.set_title(f"2. Final 32QAM Constellation\nSNR: {snr_vals[0]:.2f} dB")

_ax2.axis('equal')

_ax2.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

_ax2.grid(True, linestyle=':', alpha=0.6)

_phase_diff = np.unwrap(np.angle(rx_cpr_all[-2000:, 0] * np.conj(rx_eq_cut[-2000:, 0])))

_ax3 = plt.subplot(1, 3, 3)

_ax3.plot(_phase_diff, color='tab:orange', linewidth=1)

_ax3.set_title("3. DD-PLL Phase Error Residual")

_ax3.set_xlabel("Symbols"); _ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

mo.vstack([performance_report, mo.as_html(fig_diag_np)])

return

if __name__ == "__main__":

app.run()