import os

import time

import jax

import jax.numpy as jnp

import matplotlib.pyplot as plt

import scipy.signal as sp_signal

import numpy as np

# =========================================================================

# 路径配置

# =========================================================================

BASE_DIR = os.path.dirname(os.path.abspath(__file__))

FIGURE_PATH = os.path.join(BASE_DIR, 'results_32qam', 'figure')

if not os.path.exists(FIGURE_PATH):

os.makedirs(FIGURE_PATH)

# =========================================================================

# 核心评价与智能绘图函数

# =========================================================================

def get_snr(ref, rec):

rec = rec / (jnp.sqrt(jnp.mean(jnp.abs(rec)**2)) + 1e-12)

ref = ref / (jnp.sqrt(jnp.mean(jnp.abs(ref)**2)) + 1e-12)

best_snr = -100.0

best_rec = rec

for shift in range(-5, 6):

r_sh = jnp.roll(rec, shift)

# 32QAM 依然具有 90 度旋转对称性

for angle in [0, jnp.pi/2, jnp.pi, 3*jnp.pi/2]:

temp_rec = r_sh * jnp.exp(-1j * angle)

phase_offset = jnp.angle(jnp.mean(temp_rec * jnp.conj(ref)))

temp_rec_final = temp_rec * jnp.exp(-1j * phase_offset)

mse = jnp.mean(jnp.abs(ref - temp_rec_final)**2)

snr = 10 * jnp.log10(1.0 / (mse + 1e-12))

if snr > best_snr:

best_snr = snr

best_rec = temp_rec_final

return best_snr, best_rec

def plot_psd(sig, fs, baud_rate, beta, noise_var, name='Signal PSD', filename='psd_analysis.png'):

"""自动适配 2 SPS (滚降) 和 1 SPS (混叠平坦) 的智能 PSD 绘图函数"""

plt.figure(figsize=(10, 5))

for i in range(sig.shape[1]):

f, Pxx_den = sp_signal.welch(np.array(sig[:, i]), fs, nperseg=1024, return_onesided=False)

f = np.fft.fftshift(f)

Pxx_den = np.fft.fftshift(Pxx_den)

plt.semilogy(f / 1e9, Pxx_den, alpha=0.7, label=f'Simulated Pol {i}')

f_theo = np.linspace(-fs/2, fs/2, 2048)

Ts = 1.0 / baud_rate

f_abs = np.abs(f_theo)

psd_theo = np.zeros_like(f_theo)

if np.isclose(fs, baud_rate * 2):

f1 = (1.0 - beta) / (2.0 * Ts)

f2 = (1.0 + beta) / (2.0 * Ts)

idx_pass = f_abs <= f1

psd_theo[idx_pass] = Ts

idx_trans = (f_abs > f1) & (f_abs <= f2)

psd_theo[idx_trans] = (Ts / 2.0) * (1.0 + np.cos((np.pi * Ts / beta) * (f_abs[idx_trans] - f1)))

P_avg = 0.5

noise_psd = noise_var / fs

psd_theo = (psd_theo * P_avg) + noise_psd

label_theo = 'Theoretical RRC + AWGN Floor'

plt.ylim(bottom=1e-14, top=1e-9)

elif np.isclose(fs, baud_rate):

psd_theo[:] = 1.0 / baud_rate

label_theo = 'Theoretical Flat Spectrum (1 SPS Aliased)'

plt.ylim(bottom=1e-12, top=1e-9)

else:

psd_theo[:] = np.nan

label_theo = 'Unknown SPS'

if not np.isnan(psd_theo).all():

plt.semilogy(f_theo / 1e9, psd_theo, 'k--', linewidth=2, label=label_theo)

plt.title(name)

plt.xlabel('Frequency (GHz)')

plt.ylabel('Power Spectral Density (V^2/Hz)')

plt.grid(True, which='both', linestyle='--', alpha=0.5)

plt.legend()

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, filename), dpi=300)

plt.close()

def plot_eye(sig, sps=2, name='Eye Diagram', amplitude_limit=2.0, filename='eye_diagram.png'):

plt.figure(figsize=(12, 5))

num_points = int(2 * sps) + 1

num_traces = 800

for i in range(2):

ax = plt.subplot(1, 2, i+1)

start = 20000 if sig.shape[0] > 20000 else 0

data = np.array(sig[start:, i].real)

traces = []

for j in range(0, min(len(data) - num_points, num_traces * sps), sps):

traces.append(data[j : j + num_points])

if len(traces) > 0:

t = np.linspace(0, 2, num_points)

for trace in traces:

ax.plot(t, trace, 'b-', alpha=0.05, linewidth=0.5)

ax.axvline(x=1.0, color='r', linestyle='--', alpha=0.3, linewidth=1)

ax.set_xlim([0, 2])

ax.set_ylim([-amplitude_limit, amplitude_limit])

ax.set_title(f'{name} - Pol {i}')

ax.set_xlabel('Time (Symbol Period)')

ax.set_ylabel('Amplitude')

ax.grid(True, linestyle='--', alpha=0.5)

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, filename), dpi=300)

plt.close()

# =========================================================================

# 物理损伤模块

# =========================================================================

def apply_chromatic_dispersion(sig, baud_rate, sps, D=17.0, L=80.0, lambda_=1550e-9):

fs = baud_rate * sps

c = 299792458.0

beta2 = -(lambda_**2) / (2 * jnp.pi * c) * (D * 1e-6)

f = jnp.fft.fftfreq(sig.shape[0], 1/fs)

omega = 2 * jnp.pi * f

H = jnp.exp(-1j * (beta2 / 2) * (L * 1e3) * omega**2)

sig_f = jnp.fft.fft(sig, axis=0)

return jnp.fft.ifft(sig_f * H[:, None], axis=0)

def get_safe_rrc_taps(sps, num_taps, beta=0.1):

t = np.arange(num_taps) - (num_taps - 1) / 2.0

t = t / sps

h = np.zeros_like(t)

for i, val in enumerate(t):

if val == 0.0:

h[i] = 1.0 - beta + (4 * beta / np.pi)

elif np.abs(np.abs(val) - 1.0 / (4 * beta)) < 1e-6:

h[i] = (beta / jnp.sqrt(2)) * ((1 + 2 / np.pi) * jnp.sin(np.pi / (4 * beta)) +

(1 - 2 / np.pi) * jnp.cos(np.pi / (4 * beta)))

else:

num = jnp.sin(np.pi * val * (1 - beta)) + 4 * beta * val * jnp.cos(np.pi * val * (1 + beta))

den = np.pi * val * (1 - (4 * beta * val)**2)

h[i] = num / den

return h / jnp.sqrt(jnp.sum(h**2))

# =========================================================================

# 魔法发生地：专为 32QAM 打造的 JAX 2SPS 进出 DD-LMS 分数间隔均衡器

# =========================================================================

@jax.jit(static_argnames=['taps', 'train_len'])

def fse_2sps_qam32_lms_equalizer(rx_2sps, tx_train, const, taps=7, lr_da=2e-3, lr_dd=5e-4, train_len=10000):

n_samples = rx_2sps.shape[0]

pad = taps // 2

rx_pad = jnp.pad(rx_2sps, ((pad, pad), (0, 0)))

def step(W, i):

# 滤波器以 2 SPS 速率滑动

x = jax.lax.dynamic_slice(rx_pad, (i, 0), (taps, 2))

x = jnp.flip(x, axis=0)

y0 = jnp.sum(W[0, 0] * x[:, 0] + W[0, 1] * x[:, 1])

y1 = jnp.sum(W[1, 0] * x[:, 0] + W[1, 1] * x[:, 1])

y = jnp.array([y0, y1])

# 将采样点索引 i 映射到符号索引 sym_idx

sym_idx = i // 2

d_da = tx_train[sym_idx]

# 32QAM 密集二维复平面距离判决

dist0 = jnp.abs(y0 - const)

dist1 = jnp.abs(y1 - const)

d_dd = jnp.array([const[jnp.argmin(dist0)], const[jnp.argmin(dist1)]])

is_train = sym_idx < train_len

d = jnp.where(is_train, d_da, d_dd)

lr_base = jnp.where(is_train, lr_da, lr_dd)

e = d - y

# 核心逻辑：只在偶数索引（最佳采样点）更新权重！奇数点权重冻结。

is_even = (i % 2 == 0)

lr = jnp.where(is_even, lr_base, 0.0)

W = W.at[0, 0].add(lr * e[0] * jnp.conj(x[:, 0]))

W = W.at[0, 1].add(lr * e[0] * jnp.conj(x[:, 1]))

W = W.at[1, 0].add(lr * e[1] * jnp.conj(x[:, 0]))

W = W.at[1, 1].add(lr * e[1] * jnp.conj(x[:, 1]))

return W, y

W0 = jnp.zeros((2, 2, taps), dtype=jnp.complex64)

W0 = W0.at[0, 0, taps//2].set(1.0)

W0 = W0.at[1, 1, taps//2].set(1.0)

_, rx_eq_2sps = jax.lax.scan(step, W0, jnp.arange(n_samples))

return rx_eq_2sps

@jax.jit

def dd_pll(rx_syms, const, mu=0.015):

"""判决引导锁相环 (Decision-Directed PLL)，专门对付十字形 32QAM"""

def step(phi, y_in):

y_rot = y_in * jnp.exp(-1j * phi)

idx = jnp.argmin(jnp.abs(y_rot - const))

d = const[idx]

err = jnp.angle(y_rot * jnp.conj(d))

phi_next = phi + mu * err

return phi_next, y_rot

_, rx_out = jax.lax.scan(step, 0.0, rx_syms)

return rx_out

# =========================================================================

# 主仿真逻辑

# =========================================================================

def main():

start_time = time.time()

print("Step 1: 32QAM Signal Generation & Pulse Shaping...")

num_syms = 131072

baud_rate = 32e9

beta = 0.1

# 手动精细生成 32QAM 十字形星座图 (Cross-QAM)

grid = np.array([-5, -3, -1, 1, 3, 5])

X, Y = np.meshgrid(grid, grid)

c_raw = (X + 1j*Y).flatten()

corners = [5+5j, 5-5j, -5+5j, -5-5j]

c_32qam = np.array([pt for pt in c_raw if pt not in corners])

const = jnp.array(c_32qam)

const = const / jnp.sqrt(jnp.mean(jnp.abs(const)**2)) # 严格归一化

key = jax.random.PRNGKey(42)

tx_indices = jax.random.randint(key, (num_syms, 2), 0, 32)

tx_syms = const[tx_indices]

rrc_taps_len = 65

h_rrc = get_safe_rrc_taps(sps=2, num_taps=rrc_taps_len, beta=beta)

tx_sig = sp_signal.upfirdn(np.array(h_rrc), np.array(tx_syms), up=2, axis=0)

delay = (rrc_taps_len - 1) // 2

rx_2sps = jnp.array(tx_sig[delay : delay + num_syms * 2])

print("Step 2: Realistic Channel Impairments...")

theta = jnp.pi / 4 * 0.6

J_rot = jnp.array([[jnp.cos(theta), jnp.sin(theta)],

[-jnp.sin(theta), jnp.cos(theta)]])

rx_2sps = rx_2sps @ J_rot.T

lw = 10e3

Ts = 1.0 / (baud_rate * 2)

sigma = jnp.sqrt(2 * jnp.pi * lw * Ts)

phase_steps = sigma * jax.random.normal(jax.random.PRNGKey(99), (rx_2sps.shape[0], 1))

rx_2sps = rx_2sps * jnp.exp(1j * jnp.cumsum(phase_steps, axis=0))

rx_2sps = apply_chromatic_dispersion(rx_2sps, baud_rate, sps=2, D=17.0, L=80.0)

# 精确计算噪声功率用于理论公式

noise_amp = 0.02

noise_variance = 2 * (noise_amp**2)

noise = (jax.random.normal(jax.random.PRNGKey(123), rx_2sps.shape) +

1j * jax.random.normal(jax.random.PRNGKey(124), rx_2sps.shape)) * noise_amp

rx_2sps += noise

print("Generating PSD Plot Before DSP (2 SPS)...")

plot_psd(rx_2sps, fs=baud_rate*2, baud_rate=baud_rate, beta=beta, noise_var=noise_variance,

name='1. PSD Before DSP (2 SPS)', filename='01_psd_before_dsp.png')

print("Step 2.5: Static CDC & Rx Matched Filtering...")

rx_2sps = apply_chromatic_dispersion(rx_2sps, baud_rate, sps=2, D=17.0, L=-80.0)

rx_mf = sp_signal.upfirdn(np.array(h_rrc), np.array(rx_2sps), up=1, axis=0)

rx_2sps_mf = jnp.array(rx_mf[delay : delay + num_syms * 2])

print(f"Step 3: 32QAM Fractionally Spaced Equalization (FSE) ({num_syms} symbols!)...")

rx_2sps_mf = rx_2sps_mf / jnp.sqrt(jnp.mean(jnp.abs(rx_2sps_mf)**2, axis=0))

# 使用重写的 2 SPS 进/出 DD-LMS 均衡器

rx_eq_2sps = fse_2sps_qam32_lms_equalizer(rx_2sps_mf, tx_syms, const, taps=7, train_len=10000)

rx_eq_2sps = rx_eq_2sps / jnp.sqrt(jnp.mean(jnp.abs(rx_eq_2sps)**2, axis=0))

print("Generating PSD Plot After FSE (Still 2 SPS)...")

plot_psd(rx_eq_2sps[10000:], fs=baud_rate*2, baud_rate=baud_rate, beta=beta, noise_var=noise_variance,

name='2. PSD After Equalization (2 SPS)', filename='02_psd_after_fse.png')

print("Step 4: Downsampling to 1 SPS & Convergence Normalization...")

# 均衡完成，提取最佳采样点交给锁相环

rx_1sps_eq = rx_eq_2sps[::2]

start_idx = 15000

rx_eq_cut = rx_1sps_eq[start_idx:]

tx_final = tx_syms[start_idx:start_idx + rx_eq_cut.shape[0]]

print("Step 5: Decision-Directed Carrier Phase Recovery (DD-PLL)...")

rx_cpr_list = []

for i in range(2):

pol_cpr_out = dd_pll(rx_eq_cut[:, i], const, mu=0.015)

rx_cpr_list.append(pol_cpr_out)

rx_cpr_all = jnp.stack(rx_cpr_list, axis=1)

print("Generating Final PSD Plot (1 SPS Aliased)...")

plot_psd(rx_aligned_all if 'rx_aligned_all' in locals() else rx_cpr_all,

fs=baud_rate, baud_rate=baud_rate, beta=beta, noise_var=noise_variance,

name='3. Final Aliased PSD (1 SPS)', filename='03_psd_final_aliased.png')

print("Step 6: Performance Evaluation & Alignment...")

aligned_list = []

snr_vals = []

for i in range(2):

snr_v, aligned_sig = get_snr(tx_final[:, i], rx_cpr_all[:, i])

print(f"   Pol {i} Final SNR: {snr_v:.2f} dB")

aligned_list.append(aligned_sig)

snr_vals.append(snr_v)

rx_aligned_all = jnp.stack(aligned_list, axis=1)

print("Generating Final DSP Plots & Diagnostics...")

plt.figure(figsize=(15, 5))

ax1 = plt.subplot(1, 3, 1)

ax1.scatter(rx_eq_cut[-8000:, 0].real, rx_eq_cut[-8000:, 0].imag, s=1, alpha=0.3, color='tab:blue')

ax1.set_title("1. After FSE (Ring Structure)")

ax1.axis('equal'); ax1.grid(True, linestyle=':', alpha=0.6)

ax2 = plt.subplot(1, 3, 2)

ax2.scatter(rx_aligned_all[-8000:, 0].real, rx_aligned_all[-8000:, 0].imag, s=1, alpha=0.3, color='tab:green')

ax2.set_title(f"2. Final 32QAM\nSNR: {float(snr_vals[0]):.2f} dB")

ax2.axis('equal')

ax2.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

ax2.grid(True, linestyle=':', alpha=0.6)

phase_diff = jnp.unwrap(jnp.angle(rx_cpr_all[-2000:, 0] * jnp.conj(rx_eq_cut[-2000:, 0])))

ax3 = plt.subplot(1, 3, 3)

ax3.plot(phase_diff, color='tab:orange', linewidth=1)

ax3.set_title("3. DD-PLL Phase Track")

ax3.set_xlabel("Symbols"); ax3.grid(True, linestyle=':', alpha=0.6)

plt.tight_layout()

plt.savefig(os.path.join(FIGURE_PATH, '04_diagnostic_plot.png'), dpi=300)

plt.close()

plt.figure(figsize=(10, 5))

for i in range(2):

ax = plt.subplot(1, 2, i+1)

ax.scatter(rx_aligned_all[-8000:, i].real, rx_aligned_all[-8000:, i].imag, s=1, alpha=0.3)

ax.set_title(f'32QAM Constellation Pol {i}\nSNR: {float(snr_vals[i]):.2f} dB')

ax.axis('equal')

ax.scatter(const.real, const.imag, c='red', marker='x', s=30, alpha=0.9)

ax.grid(True, linestyle=':', alpha=0.5)

plt.savefig(os.path.join(FIGURE_PATH, '05_constellations.png'), dpi=300)

plt.close()

end_time = time.time()

print("\n" + "="*55)

print(f"32QAM 高级 FSE 仿真结束")

print(f"处理数据量: {num_syms} 个符号")

print(f"总共耗时: {end_time - start_time:.2f} 秒")

print(f"图像已保存至目录:\n   {FIGURE_PATH}\n")

print("本次生成的图像文件清单:")

if os.path.exists(FIGURE_PATH):

saved_files = [f for f in os.listdir(FIGURE_PATH) if f.endswith('.png')]

for idx, file_name in enumerate(sorted(saved_files), 1):

print(f"  [{idx}] {file_name}")

print("="*55 + "\n")

if __name__ == "__main__":

main()